A novel composite by coating acorn shell (Quercus vulcanica) with Chitosan  for removal of Cr (VI) from aqueous solution

Şerife PARLAYICI 1, Elif ÖZDEMİR 2, Kübra TUNA 1, Erol PEHLİVAN 1

1 Konya Teknik Üniversitesi, Kimya Mühendisliği Bölümü, Kampüs, 42079 Konya, Türkiye (serife842@hotmail.com), (kubratnaa@gmail.com), (erolpehlivan@gmail.com)

2 Department of Chemical Engineering, Selcuk University, Campus, 42079 Konya, Turkey, (ozdmrelif1248@gmail.com)

ABSTRACT

Water contamination raised with rapid and irregular industrialization growing, so elimination of  Cr (VI) ions from contaminated water before discharge is vitally important.  The purpose of this study;  using acorn shell (AS) and Chitosan-coated acorn shell (CtsAS) as a biomaterial in the removal of Cr (VI) from the aqueous solutions. CtsAS composite was prepared by adding glutaraldehyde as a cross-linking agent and operational feasibility of CtsAS for  Cr (VI) biosorption from aqueous environment was investigated by using different kinds of factors. The effects of contact time, initial solution pH, initial Cr (VI) concentration, and biosorbent amount on the elimination of Cr (VI) for AS and CtsAS composite were investigated by batch experiments. The biosorption system was tested using various isotherm models. The optimum conditions for the removal of Cr (VI) were found to be the amount of biosorbent 0.1 g, pH  2.0 and equilibrium time of 120 min for AS. On the other hand, the optimal conditions for the biosorption  of Cr (VI) by CtsAS were found to be: 2.0-2.1 initial pH, 90 min of exposure duration, 0.05 g biosorbent amount, 150 mg/L of initial Cr (VI) concentration at 25 oC. The maximum biosorption capacity calculated from the Langmiur was found to be 89.29 mg/g and  44.25 mg/g for CtsAS and AS, respectively. The characterization of CtsAS and AS were performed using two techniques; Scanning Electron Microscope (SEM) and Fourier Transform Infrared Spectroscopy (FT-IR). The produced a novel CtsAS composite is offering an alternative biosorbent for Cr(VI) removal from aqueous mediums.

Keywords: Biosorption; Composite; Chitosan; Acorn shell; Cr (VI)